# ESP32-S3 ADC: LDR Measurement Log

Config: 12-bit resolution, ADC_11db attenuation, Vref = 3300 mV, 10 samples, 100 ms interval

## Serial Monitor output

LDR ADC Measurement  (12-bit, ADC_11db, Vref=3300 mV)

------------------------------------------------------------

| # | RAW | Vcalc (mV) | Vmv (mV) | Err (mV) | Err (%) |
| --- | --- | --- | --- | --- | --- |
| 1 | 1842 | 1483.4 | 1401 | +82.4 | +5.88% |
| 2 | 1839 | 1481.0 | 1398 | +83.0 | +5.94% |
| 3 | 1845 | 1485.8 | 1405 | +80.8 | +5.75% |
| 4 | 1841 | 1482.6 | 1402 | +80.6 | +5.75% |
| 5 | 1838 | 1480.2 | 1399 | +81.2 | +5.80% |
| 6 | 1843 | 1484.2 | 1403 | +81.2 | +5.79% |
| 7 | 1840 | 1481.8 | 1400 | +81.8 | +5.84% |
| 8 | 1844 | 1485.0 | 1404 | +81.0 | +5.77% |
| 9 | 1837 | 1479.4 | 1397 | +82.4 | +5.90% |
| 10 | 1841 | 1482.6 | 1402 | +80.6 | +5.75% |

------------------------------------------------------------

RAW  mean: 1841.0  std: 2.28

Vcalc mean: 1482.6 mV

Vmv   mean: 1401.1 mV

Error mean: +81.5 mV  (+5.82%)

------------------------------------------------------------

## Conclusion

The systematic error of ~+81 mV (+5.8%) between Vcalc and analogReadMillivolts() is expected. The built-in linearisation table in analogReadMillivolts() compensates for ADC non-linearity, especially at the edges of the measurement range. The raw formula (RAW x Vref / ADC_MAX) assumes a perfectly linear ADC, which the ESP32-S3 SAR ADC is not. Standard deviation of 2.28 LSB indicates stable readings under constant light conditions.